# Prawler Firmware

# System Description and Diagram

## Block Diagram

# RTC

The RTC subsystem drives the external AB-RTCMC-32.768kHz-EOA9-S3 over SPI2. It provides wall-clock time, Unix epoch conversion, FAT timestamping, recorder filename timestamps, shell diagnostics, and the external countdown wake source used by STOP2 low-power cycling.

The driver includes register access, date/time, alarm, countdown timer, interrupt flag, temperature, EEPROM, RAM, calibration, and epoch conversion.

## Hardware interface

| Item | Firmware binding |
| --- | --- |
| Device | AB-RTCMC-32.768kHz-EOA9-S3 external RTC |
| MCU SPI | SPI2 / hspi2 |
| SPI config | Master, 8-bit, CPOL low, CPHA first edge, MSB first, software NSS, prescaler 32 |
| SCK/MISO/MOSI | PB13 / PB14 / PB15 |
| Chip select / CE | PB12 / SPI2_CS_Pin; driver selects high and deselects low |
| Interrupt | PB10 / RTC_INTod_Pin; falling-edge GPIO event with pull-up |
| Clock-output enable | PC6 / CLK_OE_Pin; low in current idle/disabled-clock-output handling |

RTC_Init() is called at boot after SPI2 initialization.

Boot sets g_app.rtc_status = PERIPH_READY on RTC_OK, otherwise PERIPH_ERROR and prints the RTC error code.

## Register model

The header maps RTC pages to symbolic register addresses:

| Page | Register range | Firmware use |
| --- | --- | --- |
| Control | 0x00-0x04 | Write enable, timer enable, interrupt enables, flags, status, reset. |
| Clock | 0x08-0x0E | Seconds, minutes, hours, day, weekday, month, year. |
| Alarm | 0x10-0x16 | Alarm match values and per-field enable bits. |
| Timer | 0x18-0x19 | 16-bit countdown timer. |
| Temperature | 0x20 | RTC temperature. |
| EEPROM | 0x28-0x33 | User bytes and control/calibration fields. |
| RAM | 0x38-0x3F | RTC RAM bytes. |

The current SPI command convention is read command: ’0x80 | reg’ and write command ’0x00 | reg’. Register reads transmit command plus one dummy byte and use the second received byte as data. Multi-register reads loop through RTC_ReadRegister().

## Initialization sequence

RTC_Init() performs this sequence:

- Clears the driver pending-interrupt flag.

- Drives CLKOE low.

- Deselects the RTC by driving PB12 low.

- Delays 10 ms.

- Enables writes in Control_1.

- Clears all supported flags.

- Writes Control_INT as 0x00 to disable RTC interrupts initially.

- Reads EEPROM control, clears trickle-charge bits, configures 1 Hz clock-output frequency bits, enables temperature compensation, writes EEPROM control back, and waits for EEPROM ready.

- Calls RTC_EnableClockOutput(false).

The status enum is RTC_OK, RTC_ERROR, RTC_TIMEOUT, RTC_INVALID_PARAM, and RTC_EEPROM_BUSY.

## Time/date API and epoch conversion

RTC_DateTime_t stores binary values. The driver converts between binary and RTC BCD registers. The public year field is 0..79, representing years 2000-2079. Weekday is intentionally unused by current firmware; 24-hour mode is assumed by RTC_GetDateTime().

| Function | Behavior |
| --- | --- |
| RTC_SetDateTime() | Validates time/date fields, enables writes, writes seconds/minutes/hours/day/month/year in BCD. |
| RTC_GetDateTime() | Reads clock registers, masks status/control bits, converts BCD to binary. |
| RTC_ToUnixEpoch() | Converts RTC date/time to Unix epoch seconds. |
| RTC_FromUnixEpoch() | Converts Unix epoch seconds to RTC date/time fields. |
| RTC_ToGPSEpoch() | Converts date/time using the driver's GPS epoch helper. |

Shell commands expose these paths: ’settime <UNIX_EPOCH>’, ’rtc-settime YYYY MM DD HH MM SS’, ’rtc-time, rtc-epoch’, and ’rtc-temp’. Handlers wrap RTC access with lowpower_rtc_begin() / lowpower_rtc_end() so SPI2 is temporarily restored when profile peripherals are not already up.

## FAT and recorder integration

FatFS uses get_fattime() from FATFS/App/fatfs.c. That function calls RTC_GetDateTime() and packs a FAT timestamp; on RTC read failure it returns zero. The recorder uses RTC_GetDateTime() through get_unix_timestamp() to timestamp each measurement and uses RTC_FromUnixEpoch() in build_filename() to build DDMMYYYY_HHMMSS_record.csv.

## Alarm, timer, and interrupt APIs

The driver supports basic alarms, extended alarms, countdown timers, and interrupt flag handling:

| Area | Main functions |
| --- | --- |
| Basic alarm | RTC_SetAlarm(), RTC_GetAlarm(), RTC_EnableAlarm(), RTC_ClearAlarmFlag() |
| Extended alarm | RTC_SetExtendedAlarm(), RTC_GetExtendedAlarm() |
| Timer | RTC_SetTimer(), RTC_GetTimer(), RTC_EnableTimer(), RTC_EnableTimerInterrupt(), RTC_SetTimerDivision() |
| Interrupt flags | RTC_GetInterruptFlags(), RTC_GetStatusFlags(), RTC_GetInterruptState(), RTC_ClearInterruptSources(), RTC_ClearAllFlags() |
| Pending/line state | RTC_NotifyInterrupt(), RTC_IsInterruptPending(), RTC_ClearPendingInterrupt(), RTC_IsInterruptAsserted() |

Timer source constants are 32 Hz, 8 Hz, 1 Hz, and 0.5 Hz. STOP2 wake uses a 20-second one-shot timer with 1 Hz division and timer interrupt. Shell timer commands are rtc-timer-set, rtc-timer-stop, and rtc-timer-status.

# SD Card/FATFS

The storage subsystem provides SD-card-backed FAT file access over SPI1. It combines the low-level SPI SD/MMC block driver, Cube/FatFS diskio wrapper, FatFS application glue and timestamp callback, project-level filesystem wrapper, and low-power SD/SPI ownership.

Profile recordings are buffered in RAM while sampling. The SD card is powered and mounted only for final CSV flush. Shell filesystem commands also power/mount the card only around the requested operation.

## Hardware Interface

| Item | Firmware binding |
| --- | --- |
| MCU SPI | SPI1 / hspi1 |
| SPI config | Master, 8-bit, CPOL low, CPHA first edge, MSB first, software NSS |
| Boot/power-up prescaler | SPI_BAUDRATEPRESCALER_64 |
| Driver slow/fast macros | slow prescaler 64, fast prescaler 4 |
| SCK/MISO/MOSI | PA5 / PA6 / PA7 |
| Chip select | PA4 / SPI1_CS_Pin, idle high, selected low |
| SD power | PB2 / SD_PWR_Pin, active high |
| FatFS drive | 0: |

main() initializes SPI1 and FatFS, calls filesystem_init(), powers the SD/SPI path up, attempts a boot mount, records SD status, unmounts on successful boot check, and powers the SD/SPI path back down (Core/Src/main.c).

## Low-level SD SPI driver

sd_spi.c is a driver derived from the reference SPI-mode FATFS SD implementation. It owns the cached disk status Stat, card type flags, command transmit path, data block receive/write paths, and a structured diagnostic record SD_SPI_Diag_t.

USER_SPI_initialize() accepts only drive 0. It sends 80 dummy clocks, sends CMD0, probes SD v2 with CMD8/OCR, initializes with ACMD41 and CMD58 when appropriate, falls back to SD v1/MMC handling, sets 512-byte block length for byte-addressed cards, switches to fast clock on success, clears STA_NOINIT, and records diagnostic stages throughout.

USER_SPI_read() supports single-sector CMD17 and multi-sector CMD18 reads. USER_SPI_write() supports single-sector CMD24 and multi-sector CMD25 writes when _USE_WRITE == 1. USER_SPI_ioctl() implements synchronization, sector count, sector size, erase block size, and trim paths; GET_SECTOR_SIZE returns 512.

### Diskio and FatFS

FATFS/Target/user_diskio.c delegates FatFS disk operations to the SD SPI driver: initialize, status, read, write, and ioctl call USER_SPI_* functions. USER_diskio_reset() calls USER_SPI_reset() and clears FatFS driver's initialized flags for USER_Driver entries. The low-power manager calls this reset when SD power is removed .

MX_FATFS_Init() links USER_Driver to USERPath. get_fattime() reads the external RTC and returns the packed FAT timestamp; RTC read failure returns 0. File modification times therefore depend on a valid external RTC.

## Filesystem wrapper API

filesystem.c owns one FATFS object, a shell file object, a recorder log file object, current path 0:, last FRESULT, a mounted flag, and shell scratch buffers.

| Function | Behavior |
| --- | --- |
| filesystem_init | Clears buffers and forces reset. |
| filesystem_mount | Calls f_mount(&fs, "0:", 1) unless already mounted. |
| filesystem_unmount | Calls f_mount(NULL, "0:", 0) when mounted. |
| filesystem_force_reset | Unregisters FatFS work area, clears file objects/state, resets path to 0:, clears mounted flag. |
| filesystem_ls | Opens current path, formats directory entries, closes directory. |
| filesystem_cat | Opens a file read-only and streams chunks through a callback. |
| filesystem_log_create | Opens recorder log with `FA_WRITE |
| filesystem_log_write | Writes bytes and requires full byte count. |
| filesystem_log_sync | Flushes the open recorder log. |
| filesystem_log_close | Closes the recorder log. |
| filesystem_find_latest | Finds newest file matching a suffix using FAT date/time fields. |

FS_Result_t maps key FatFS errors to FS_FILE_NOT_FOUND, FS_ACCESS_DENIED, or generic FS_ERROR while preserving raw FRESULT for diagnostics.

## Recorder flush behavior

During profile acquisition, measurements remain in RAM. At normalization completion, sd_mount_for_flush() powers SD/SPI down, waits 20 ms, powers up, and attempts filesystem_mount(). If the first mount fails, it logs both wrapper and raw FatFS result, repeats the down/up cycle, and retries. Sustained failure marks SD error, powers the SD path down, and builds realtime idata from RAM.

flush_profile_to_sd() builds DDMMYYYY_HHMMSS_record.csv, creates it with FA_CREATE_NEW, writes the fixed CSV header, writes each buffered measurement row, syncs, and closes. The CSV header is:

MeasurementNo,Unix_Epoch_UTC,CTD_C,CTD_T,CTD_D,Optode_O2,Optode_AirSat,Optode_Temp,Optode_Cal_Ph,Optode_Tc_Ph,Optode_C1_Ph,Optode_C2_Ph,Optode_C1_Amp,Optode_C2_Amp,Optode_Temp_raw,Wetlab_C1_lambda,Wetlab_C1_signal,Wetlab_C2_lambda,Wetlab_C2_signal,Wetlab_C3_lambda,Wetlab_C3_signal,Wetlab_Therm\r\n

The recorder formats floats with %f, WetLab fields as unsigned integers, and terminates each row with \r\n.

## Shell command behavior

| Command | Behavior |
| --- | --- |
| fs-mount | Powers SD/SPI up, mounts, reports success or diagnostic failure, unmounts on success, powers SD/SPI down. |
| fs-unmount | Calls filesystem_unmount() and powers SD/SPI down. |
| fs-ls | Powers SD/SPI up, mounts, lists root path, unmounts, powers SD down. |

Filesystem shell commands run in deferred task context. During active profile peripheral ownership, SD access commands report that SD access is unavailable during a profile.

# CTD

The CTD driver owns the RS-232/UART transaction used to wake the CTD, issue a ts command, receive the returned text, and parse pressure, temperature, and conductivity into ctd_data_t. The shell uses the blocking helpers through ctd and ctd-raw; profile acquisition uses the split-phase ctd_fire() / ctd_collect() path through sensors_sample().

The driver is module-stateful. It stores the active UART handle, TX/RX completion flags, the most recent Receive-To-Idle length, and a static rx_buf[CTD_RX_BUF_SIZE] buffer. Current CTD RX buffer size is 1024 bytes.

## Hardware interface

| Item | Current firmware binding |
| --- | --- |
| MCU UART | USART3 / huart3 |
| Baud/config | 9600 baud, 8N1, no hardware flow control |
| TX pin | PC4, USART3 TX |
| RX pin | PC5, USART3 RX |
| TX DMA | DMA1 Channel 2, normal mode, memory-to-peripheral |
| RX DMA | DMA1 Channel 3, normal mode, peripheral-to-memory |
| RS-232 transceiver enable | PB0 / PB0_USART3_EN_Pin, active high |
| Init call | ctd_init(&huart3) in main() and lowpower_ctd_uart_up() |

main() initializes USART3, calls ctd_init(&huart3), and marks g_app.ctd_status = PERIPH_READY. Low-power restore repeats the same binding: lowpower_ctd_uart_up() initializes huart3, drives PB0 high, calls ctd_init(&huart3), and marks CTD ready. Low-power shutdown aborts/deinitializes huart3, drives PB0 low, parks PC4/PC5 as analog pulldown, and marks CTD off.

## Public API

| Function | Role |
| --- | --- |
| ctd_init | Stores the UART handle used by all CTD transactions. |
| ctd_wakeup | Sends wake bytes and confirms any CTD response. |
| ctd_ts | Blocking parsed sample helper for shell use. |
| ctd_sample_raw | Wake, issue ts\r, collect raw response, optionally copy NUL-terminated text, and return byte count. |
| ctd_fire | Split-phase fire: reset UART, start continuous RX DMA, send ts\r, and leave RX active. |
| ctd_collect | Split-phase collect: compute received bytes from DMA NDTR, stop RX, NUL-terminate, parse. |
| ctd_notify_tx_cplt | USART3 TX-complete fan-out target from HAL callback. |
| ctd_notify_rx_event | USART3 Receive-To-Idle fan-out target from HAL callback. |

## Wake and sample sequence

ctd_wakeup() runs up to CTD_WAKEUP_RETRIES attempts. Each attempt resets the UART, arms Receive-To-Idle DMA, sends \r\n, waits up to 1000 ms for TX completion, then waits up to CTD_WAKEUP_TIMEOUT_MS for a response. Current constants are 10 attempts and 50 ms RX wait per attempt.

ctd_sample_raw() is the normal raw/blocking sample path:

- Calls ctd_wakeup().

- Arms Receive-To-Idle DMA before transmitting the command.

- Sends ts\r by DMA TX.

- Waits up to 1000 ms for TX completion.

- Collects response chunks until it has seen a comma-containing measurement line and then the prompt character >.

- Stops after the prompt or a 10-second overall timeout.

- Aborts RX, NUL-terminates the static RX buffer, optionally copies text to the caller buffer, and returns total byte count.

The split-phase path used by sensors_sample() starts continuous RX DMA in ctd_fire() and later reads DMA progress in ctd_collect(). This lets CTD timing overlap with Optode and WetLab timing.

## Parsing and data model

ctd_data_t contains conductivity, temperature, and pressure as floats. ctd_parse_response() scans response lines separated by \r/\n, selects the first line containing a comma, and parses values in this wire order:

pressure,temperature,conductivity

The parsed struct order is different from the wire order. The recorder writes CSV columns as CTD_C, CTD_T, CTD_D, using:

m->ctd.conductivity 
m->ctd.temperature 
m->ctd.pressure

Shell output prints Conductivity, Temperature, and Pressure from the same struct fields.

## Simultaneous profile sampling

sensors_sample() runs this CTD sequence:

- WetLab power is asserted first when WetLab is enabled.

- Optode wake/fire runs when Optode is enabled.

- CTD wake runs, then ctd_fire() sends ts\r and leaves RX DMA active.

- Firmware waits WETLAB_BOOT_MS, fires WetLab if enabled, then waits SENSOR_COLLECT_WINDOW_MS.

- ctd_collect() parses the CTD data.

Current timing constants are WETLAB_BOOT_MS = 1500 ms and SENSOR_COLLECT_WINDOW_MS = 1500 ms.

## DMA and callbacks

The Cube/HAL interrupt file routes DMA and UART callbacks for CTD:

DMA1 Channel 2 services hdma_usart3_tx 
DMA1 Channel 3 services hdma_usart3_rx
HAL TX-complete calls ctd_notify_tx_cplt() when the UART instance is USART3
HAL Receive-To-Idle calls ctd_notify_rx_event() when the instance is USART3.

Receive-To-Idle paths disable DMA half-transfer interrupts after arming RX. Split-phase ctd_fire() disables half-transfer, transfer-complete, and UART error interrupt paths for the continuous DMA polling pattern before ctd_collect() reads the DMA counter.

## Shell and power behavior

ctd and ctd-raw run from deferred task handlers, not directly from the UART RX ISR (Core/Src/shell.c, Core/Src/task_handlers.c). If the profile peripheral set is down, these handlers call lowpower_sensor_interfaces_up(), wait 2000 ms, perform the read, then call lowpower_sensor_interfaces_down() and lowpower_enter_idle() (Core/Src/task_handlers.c).

# Optode

The Optode driver owns the RS-232/UART command sequence for waking the Optode communication interface, sending do sample, receiving the text response, and parsing oxygen/temperature/phase/amplitude fields into optode_data_t. Shell diagnostics use optode, optode-raw, and optode-listen; profile acquisition uses optode_wake(), optode_fire(), and optode_collect() through sensors_sample().

The driver is module-stateful. It stores the active UART handle, TX/RX completion flags, the latest RX event length, and a static rx_buf[OPTODE_RX_BUF_SIZE] buffer. Current Optode RX buffer size is 2048 bytes.

## Hardware interface

| Item | Firmware binding |
| --- | --- |
| MCU UART | USART2 / huart2 |
| Baud/config | 9600 baud, 8N1, no hardware flow control |
| TX pin | PA2, USART2 TX |
| RX pin | PA3, USART2 RX |
| RX DMA | DMA1 Channel 6, normal mode, peripheral-to-memory |
| TX DMA | DMA1 Channel 7, normal mode, memory-to-peripheral |
| RS-232 transceiver enable | PB1 / PB1_USART2_EN_Pin, active high |
| Init call | optode_init(&huart2) in main() and lowpower_optode_uart_up() |

main() initializes USART2, calls optode_init(&huart2), and marks g_app.optode_status = PERIPH_READY. 
lowpower_optode_uart_up() restores the same UART configuration, drives PB1 high, calls optode_init(&huart2), and marks the Optode ready. Shutdown aborts/deinitializes huart2, drives PB1 low, parks PA2/PA3 as analog pulldown, and marks Optode off.

## Public API

| Function | Role |
| --- | --- |
| optode_init | Stores the UART handle used by all Optode transactions. |
| optode_sample | Blocking parsed sample helper. |
| optode_sample_raw | Wake, issue do sample\r\n, collect raw response, optionally copy NUL-terminated text, return byte count. |
| optode_wake | Split-phase wake helper that sends ignored semicolon command/comment lines. |
| optode_fire | Split-phase fire helper that starts continuous RX DMA and sends do sample\r\n. |
| optode_collect | Split-phase collect helper that reads DMA progress and parses the response. |

### Wake and sample sequence

The raw/blocking sample path in optode_sample_raw() is:

- Reset the UART and arm continuous RX DMA.

- Send the wake preamble ;;;;;;;;;;\r\n;;;;;;;;;;\r\n by DMA TX.

- Wait for TX completion and delay 150 ms.

- Abort RX and reset the UART again.

- Re-arm continuous RX DMA.

- Send do sample\r\n by DMA TX.

- Delay 1150 ms.

- Compute received length from RX DMA NDTR, abort RX, NUL-terminate the buffer, optionally copy to caller storage, and return the received length.

The split-phase path uses optode_wake() followed by optode_fire() and later optode_collect(). optode_wake() sends the same semicolon preamble and waits briefly; optode_fire() arms continuous RX DMA and sends do sample\r\n; optode_collect() reads DMA progress, stops RX, and parses.

## Data model

optode_data_t contains ten floats (Core/Inc/optode.h):

| Field | Units | CSV column |
| --- | --- | --- |
| o2_concentration | uM | Optode_O2 |
| air_saturation | percent | Optode_AirSat |
| temperature | deg C | Optode_Temp |
| cal_phase | deg | Optode_Cal_Ph |
| tc_phase | deg | Optode_Tc_Ph |
| c1_rph | deg, blue excitation phase | Optode_C1_Ph |
| c2_rph | deg, red excitation phase | Optode_C2_Ph |
| c1_amp | mV, blue excitation amplitude | Optode_C1_Amp |
| c2_amp | mV, red excitation amplitude | Optode_C2_Amp |
| raw_temp | mV, thermistor bridge voltage | Optode_Temp_raw |

## Simultaneous profile sampling

sensors_sample() calls the Optode path only when config_has_optode() is true. It wakes and fires the Optode before CTD wake/fire so the Optode can produce data during the shared collection window. After SENSOR_COLLECT_WINDOW_MS, optode_collect() parses the response.

Profile sensor levels are: CTD only (1), CTD plus Optode (2), and CTD plus Optode plus WetLab (3).

## DMA and callbacks

USART2 DMA/interrupt routing is in Core/Src/stm32l4xx_it.c: DMA1 Channel 6 services hdma_usart2_rx, DMA1 Channel 7 services hdma_usart2_tx, the HAL TX-complete callback calls optode_notify_tx_cplt() for USART2, and the HAL Receive-To-Idle callback calls optode_notify_rx_event(Size) for USART2.

Normal sample paths use continuous DMA and DMA-counter polling.

## Shell behavior

| Command | Behavior |
| --- | --- |
| optode | Powers interfaces if needed, waits 2000 ms, calls optode_sample(), prints all ten parsed fields, then powers down if it performed the power-up. |
| optode-raw | Powers interfaces if needed, waits 1300 ms, calls optode_sample_raw(), prints the raw response, then powers down if it performed the power-up. |
| sensors | Runs coordinated sampling and prints a compact Optode success or failure line when Optode is enabled. |

# WetLab

The WetLab driver owns the UART5 receive path for the WetLab ECO sensor, which auto-transmits data when powered. The driver powers the sensor path through PB4, arms UART5 RX DMA, captures text output, selects the last complete data row, and parses channel wavelengths, channel signals, and thermistor count into wetlab_data_t.

WetLab is used in three firmware paths:

- Shell diagnostics: wetlab and wetlab-raw run through deferred task handlers.

- Coordinated sensor sampling: sensors_sample() powers WetLab first, lets it boot while Optode/CTD work begins, then collects WetLab output in the shared collection window.

- Recorder output: parsed WetLab fields are stored in profile RAM and later written to the recording CSV.

The driver is module-stateful. It stores the active UART_HandleTypeDef * and a static rx_buf[1024] receive buffer.

### Hardware interface

| Item | Current firmware binding |
| --- | --- |
| Sensor | WetLab ECO sensor |
| MCU UART | UART5 / huart5 |
| Baud/config | 19200 baud, 8N1, no hardware flow control |
| TX pin | PC12, UART5 TX |
| RX pin | PD2, UART5 RX |
| RX DMA | DMA2 Channel 2, request 2, normal mode, peripheral-to-memory |
| TX DMA | none used by current WetLab driver |
| Auxiliary Select A0 | PB4 / PB4_AUX_SEL_A0_Pin, driven high for active sample window and low when off |
| Auxiliary Select A1 | PB5 / PB5_AUX_SEL_A1_Pin, kept low in low-power restore/shutdown paths |
| Driver files | Core/Src/wetlab.c, Core/Inc/wetlab.h |

main() initializes UART5, calls wetlab_init(&huart5), and marks g_app.wetlab_status = PERIPH_READY.

lowpower_wetlab_uart_up() rebuilds the UART5 handle at 19200 baud, initializes HAL UART5, keeps PB4/PB5 low after UART restore, calls wetlab_init(&huart5), and marks WetLab ready.

lowpower_wetlab_uart_down() aborts/deinitializes UART5, drives PB4/PB5 low, parks PC12 and PD2 as analog pulldown, and marks WetLab off.

## Public API

| Function | Role |
| --- | --- |
| wetlab_init | Stores the UART handle used by all WetLab RX transactions. Boot and low-power restore pass &huart5. |
| wetlab_sample | Blocking shell-style sample: power-cycle PB4, wait for boot, capture output, power down, parse last complete row. |
| wetlab_raw | Diagnostic raw capture: power PB4, capture roughly 4 seconds of output, power down, print raw bytes through shell. |
| wetlab_fire | Split-phase capture start: arm UART5 RX DMA for coordinated sampling. |
| wetlab_collect | Split-phase collect: stop DMA, select the last complete row, parse it into wetlab_data_t. |

## Wake and sample sequence

The current WetLab path receives AutoRun output rather than transmitting a command string to the sensor.

wetlab_sample() is the normal blocking shell-style sample path:

- Drive PB4 low, then high.

- Wait 1500 ms for WetLab startup/autotransmit behavior.

- Arm UART5 RX DMA.

- Wait WETLAB_SAMPLE_WAIT_MS; current value is 900 ms (Core/Inc/config.h).

- Stop RX and drive PB4 low.

- Select the last complete row and parse it.

wetlab_raw() is the raw diagnostic path:

- Drive PB4 low, then high.

- Wait 100 ms.

- Arm UART5 RX DMA.

- Capture for 4000 ms.

- Stop RX and drive PB4 low.

- Print byte count and raw text through the shell.

This path is useful for verifying row shape, boot timing, and whether the parser is rejecting data because of field count or signal-range validation.

## Parsing and data model

wetlab_data_t contains seven unsigned 16-bit values:

| Field | Meaning | Recorder CSV column |
| --- | --- | --- |
| ch1_lambda | Channel 1 excitation wavelength, nm | Wetlab_C1_lambda |
| ch1_signal | Channel 1 signal count | Wetlab_C1_signal |
| ch2_lambda | Channel 2 excitation wavelength, nm | Wetlab_C2_lambda |
| ch2_signal | Channel 2 signal count | Wetlab_C2_signal |
| ch3_lambda | Channel 3 excitation wavelength, nm | Wetlab_C3_lambda |
| ch3_signal | Channel 3 signal count | Wetlab_C3_signal |
| thermistor | Thermistor count | Wetlab_Therm |

wetlab_parse() expects one complete row in this text format:

MM/DD/YY HH:MM:SS lambda1 signal1 lambda2 signal2 lambda3 signal3 thermistor

The parser uses sscanf() and requires all 13 fields: month, day, year, hour, minute, second, three wavelength/signal pairs, and thermistor. It validates the three signal counts against the classic-mode range 0..4130; a row with any signal above 4130 is rejected. The date/time fields are parsed only to validate row shape; they are not stored in wetlab_data_t.

wetlab_find_last_row() walks the RX buffer line by line, skips CR/LF separators, ignores incomplete trailing lines, and remembers the last complete line beginning with an ASCII digit. That selected line is parsed by wetlab_parse().

The recorder stores parsed WetLab data in each measurement_data_t and writes the seven WetLab fields as unsigned integers in the same order as wetlab_data_t.

## Simultaneous profile sampling

sensors_sample() uses the WetLab split-phase path only when config_has_wetlab() is true:

- Drive PB4 high immediately so WetLab can boot while other sensors are being prepared.

- Wake/fire Optode when enabled.

- Wake/fire CTD.

- Wait WETLAB_BOOT_MS; current value is 1500 ms.

- Call wetlab_fire() to arm UART5 RX DMA.

- Wait the shared SENSOR_COLLECT_WINDOW_MS; current value is 1500 ms.

- Call wetlab_collect() to parse the last complete row.

- Drive PB4 low after collection.

The coordinated path is designed around WetLab AutoRun output rather than a command/response exchange. WetLab is active only for sensor configuration level 3 (SENSOR_CFG_ALL).

## DMA and callbacks

The driver uses polling-style DMA receive rather than Receive-To-Idle callbacks:

- wetlab_reset_uart() aborts UART5 and clears overrun, noise, and framing flags.

- wetlab_arm_rx() disables interrupts briefly around HAL_UART_Receive_DMA(), starts RX into the static 1024-byte buffer, disables DMA half-transfer and transfer-complete interrupts, clears UART error interrupt enable, then re-enables interrupts.

- wetlab_stop_rx() computes bytes received from DMA NDTR, aborts RX, NUL-terminates the buffer, removes any NUL bytes from power-on transient output by compacting non-NUL bytes left, NUL-terminates again, and returns the final byte count.

UART5 RX DMA is configured as DMA2 Channel 2, request 2, byte-aligned peripheral-to-memory, memory increment enabled, normal mode, low priority. DMA2 Channel 2 and UART5 interrupts are enabled in generated initialization.

## Shell and power behavior

wetlab, wetlab-raw, and sensors run from deferred task handlers, not directly from the UART RX ISR. If the profile peripheral set is down, these handlers call lowpower_sensor_interfaces_up(), perform the read, then call lowpower_sensor_interfaces_down() and lowpower_enter_idle().

WetLab participates in the shared sensor-interface power policy:

- Idle low-power shutdown calls lowpower_wetlab_uart_down() as part of lowpower_sensor_interfaces_down() and lowpower_idle_peripherals_down().

- Profile bring-up calls lowpower_wetlab_uart_up() through lowpower_sensor_interfaces_up().

- Profile shutdown calls lowpower_wetlab_uart_down() through lowpower_profile_peripherals_down().

- Shell commands temporarily bring interfaces up when the profile peripheral set is not already powered.

PB4 is the runtime WetLab active-sample control in current source. PB5 is held low in the low-power helper paths. New code should preserve this PB4/PB5 state model unless the auxiliary decoder/power hardware is deliberately changed.

# Main Loop Execution and Global States

## Firmware Init Sequence:

main() performs CubeMX initialization first, then project-service initialization, then enters a polling service loop. The boot sequence is:

- HAL_Init() and SystemClock_Config().

- GPIO, DMA, SPI1, SPI2, UART4, FatFS, UART5, TIM4, USART1, USART2, USART3, and IWDG initialization.

- Drive PB9 to the WiFi-off/known-state level before project services start.

- Print the startup banner on the shell UART.

- Call config_init() to initialize g_app and runtime config.

- Call transceiver_init() for the RS-232 enable pins.

- Initialize the task queue with tasker_init().

- Initialize the filesystem wrapper with filesystem_init().

- Power SD/SPI1 up, attempt a boot filesystem mount, set g_app.sd_status, unmount on success, and power SD/SPI1 down.

- Initialize the external RTC and set g_app.rtc_status.

- Initialize CTD, Optode, WetLab, recorder, and low-power manager.

- Power idle peripherals down.

- Start WiFi through lowpower_wifi_start().

- Start the UART shell with shell_init().

- Enter idle and arm the automatic low-power timer with lowpower_enter_idle() (Core/Src/main.c, Core/Src/lowpower.c).

## Main Loop Services

The main loop is intentionally simple and service-oriented:

- HAL_IWDG_Refresh() – Feed the watchdog.

- shell_task() – Handle any shell-related inputs, outputs, and task queueing.

- tasker_run() – Handle any enqueued tasks.

- recorder_service() – Active once PB8 goes high (the inductive charger disconnects), performs profile recording tasks.

- wifi_service() – Handle any wifi-shell related inputs, outputs, and task queueing.

- lowpower_service() – When in an IDLE state, this service will handle the timers and low-power sleep duty-cycle.

By default, all functionality follows the pattern of deferred execution in the main loop.

## Application Global State Variables

g_app is declared in config.h and initialized by config_init(). It contains:

| Field | Meaning |
| --- | --- |
| mode | Current Operating Mode |
| start_flag | Volatile PB8 trigger; goes HIGH when inductive charger disconnects |
| sd_status | SD/FatFS status: OFF/READY/ERROR |
| rtc_status | RTC status: OFF/READY/ERROR |
| ctd_status | CTD status: OFF/READY/ERROR |
| optode_status | Optode status: OFF/READY/ERROR |
| wetlab_status | Wetlab status: OFF/READY/ERROR |
| wifi_state | Wifi state: OFF/INIT/STREAMING/ERROR |
| sensor_level | Sensor configuration level (1-3) |

Important configuration constants live in Core/Inc/config.h. Current values are:

sample interval minimum 4 s, maximum 60 s, default 4 s
WETLAB_BOOT_MS 1500 ms
SENSOR_COLLECT_WINDOW_MS 1500 ms
WETLAB_SAMPLE_WAIT_MS 900 ms
CTD_WAKEUP_TIMEOUT_MS 50 ms
CTD_WAKEUP_RETRIES 10
OPTODE_WAKE_DELAY_MS 500 ms

# Command Shell and Task Execution

The command shell is shared by the wired USART1 console and the WiFi passthrough shell. USART1 input is received one byte at a time by interrupt into a 128-byte RX ring, then shell_task() drains that ring in the main loop and calls `shell_process_char(). WiFi input is received by UART4 interrupt into a separate 512-byte ring, then wifi_service() assembles command lines and dispatches them with shell_dispatch().

The shell command table is shell_commands[] in Core/Src/shell.c. Each entry contains a command string, a help description, and a cmd_* function pointer. ‘help’ prints this table, so adding a command table entry also adds the command to help output

Commands that can block, power hardware, or print long output are normally split into two layers:

  - cmd_* wrapper in shell.c: parses arguments, builds a small argument struct if needed, and enqueues a handler.

  - handle_* function in task_handlers.c: defers execution to tasker_run() in main-loop context.

The task queue stores function pointers plus up to TASK_ARG_SIZE bytes of copied inline arguments. Current task capacity is 16 queue slots, and the argument storage is 16 bytes per queued task (Core/Inc/tasker.h, Core/Src/tasker.c). Any command needing more than 16 bytes of argument data should not pass a large struct through tasker_enqueue(); it should store or parse the larger data elsewhere and enqueue a pointer whose lifetime is safe.

## Shell implementation rules:

- Do not call blocking functions or print from the UART RX ISR. The current ISR path only enqueues RX bytes and re-arms interrupt reception.

- Use tasker_enqueue() for command work that may block, wait on UART/SPI, power hardware, print long output, or call HAL_Delay().

- Keep task argument structs at or below 16 bytes, because tasker_enqueue() truncates copies larger than TASK_ARG_SIZE.

  - Adjust TASK_ARG_SIZE if more bytes are needed, but try to keep bigger structs off of the stack.

- Use shell_defer_prompt() before queueing a handler that will print the prompt itself later; low-power and samplerate paths already use this pattern.

- Check g_app.mode and lowpower_profile_peripherals_are_up() before accessing profile-owned peripherals from shell commands.

# Recorder Service

## Recorder State Machine

Core/Src/recorder.c owns the profile lifecycle. Internal states are:

| State | Purpose |
| --- | --- |
| REC_IDLE | Wait for debounced PB8 active-high profile request. |
| REC_BRINGUP | Bring profile peripherals up and wait PROFILE_BRINGUP_MS (3000). |
| REC_RECORDING | Sample at configured interval while PB8 stays HIGH. |
| REC_TIMEOUT | Max sample count reached; wait for PB8 LOW before normalization. |
| REC_NORMALIZING | Append normalization samples and then flush/build outputs. |

g_app.mode is set to SYS_MODE_RECORDING, SYS_MODE_TIMEOUT, or SYS_MODE_NORMALIZING around the corresponding phases.

## Trigger Contract

PB8 is active-high:

- A rising edge or pending start request begins debounce.

- PB8 must remain HIGH for DEBOUNCE_MS (100 ms) to start bring-up.

- PB8 LOW during bring-up cancels recording.

- PB8 LOW during recording transitions to normalization only after the false-start window has cleared.

- PB8 triggers during normalization are ignored and cleared.

The recorder accepts start requests from the PB8 GPIO state, pending EXTI state, and g_app.start_flag, then applies the same active-high debounce before profile bring-up.

## Recording Flow

- Reset g_profile metadata and sample counters.

- Bring up peripherals and wait for PROFILE_BRINGUP_MS.

- Take one immediate probe sample for false-start detection.

- Schedule the first recorded sample after config_get_samplerate_ms().

- Append each sample record to g_profile.measurements with a unix-epoch timestamp pulled from the RTC.

- Stop Conditions: PB8 goes LOW or the max sample count has been reached.

- Start the Optode Normalization sequence:

  - 30 samples taken, one every 20 seconds.

  - Normalization samples are appended to the profile measurements.

- Power up the SD, mount the filesystem, and write a full CSV to disk.

- Derive a realtime idata payload table in memory from the measurements.

- Return to IDLE state, restart the WiFi module, and await an ‘idata’ command to transmit the realtime data.

## False-Start Logic

FALSE_START_SAMPLES is 5, and TOLERANCE is 0.5 dbar. During the first five scheduled samples, recorder tracks max pressure. If max pressure does not exceed the initial probe depth by more than tolerance, the in-RAM profile is discarded as a false start.

A PB8 release before enough records also discards the profile.

## Timing and Limits

| Configuration | Full recorded sample cap | idata streaming row cap |
| --- | --- | --- |
| CTD only | TBD | TBD |
| CTD + Optode | TBD | TBD |
| CTD + Optode + WetLab | 680 | 500 |

For all-sensor profiles, the SD CSV can contain up to 680 measurements and the WiFi idata cache contains up to 500 rows by IDATA_CFG_ALL and RT_BUF_SIZE sizing.

## RAM Profile Layout

profile_data_t is statically allocated in recorder.c as g_profile (50KB).

Each measurement_data_t contains:

- Unix timestamp,

- CTD data,

- Optode data,

- WetLab data.

## SD CSV Output

Recorder creates a file named:

DDMMYYYY_HHMMSS_record.csv

The filename comes from g_profile.start_epoch, converted back into RTC date/time fields. flush_profile_to_sd() writes a full header line and one row per measurement in g_profile.count.

Current SD CSV columns are:

MeasurementNo, Unix_Epoch_UTC, CTD_C, CTD_T, CTD_D, Optode fields, and WetLab fields.

## Realtime Data Build

realtime_comm_build(&g_profile) is called after successful SD flush. It is also called if SD restore or flush fails after normalization, so WiFi retrieval can still work from the RAM copy.

# WiFi Communication Service

The firmware configures the onboard ISM4343-WBM-L54 module as an open WiFi access point and TCP server. The module is attached to UART4 at 9600 baud and then switched into transparent passthrough mode, so TCP client bytes are seen by firmware as UART4 bytes and firmware responses are written back over the same passthrough link.

Current compiled AP/server settings are:

| Setting | Value | Source |
| --- | --- | --- |
| AP SSID | prawler1 | WIFI_AP_SSID |
| Security | 0 | WIFI_AP_SECURITY |
| Channel | 6 | WIFI_AP_CHANNEL |
| AP IP | 192.168.10.1 | WIFI_AP_IP |
| TCP Port | 5000 | WIFI_TCP_PORT |

## Firmware AP and TCP Server Setup

The firmware AT setup sequence is:

| Step | AT Command | Function |
| --- | --- | --- |
| 1 | \r\n (empty) | Prompt sync |
| 2 | AS=0,prawler1 | Set AP SSID |
| 3 | A1=0 | Set AP security to off (no wifi encryption/password) |
| 4 | AC=6 | Set AP channel |
| 5 | Z6=192.168.10.1 | Set AP IP address |
| 6 | ZP=1,0 | Disable power-saving functions |
| 7 | AL=255 | Set DHCP lease time to max |
| 8 | AD | Start the AP |
| 9 | P2=5000 | Set TCP server access port |
| 10 | S1=1460 | Set the write-packet buffer size (max) |
| 11 | S2=50 | Set the write timeout (50ms) |
| 12 | PK=1,30000 | Enable the TCP keepalive |
| 13 | PX=0,0 | Start the passthrough-mode (becomes transparent UART) |

After PX=0,0, the module no longer returns AT prompts. wifi_setup_tcp_server() sets firmware WiFi state to WIFI_STATE_STREAMING, and wifi_service() begins treating incoming UART4 bytes as remote shell input.

## Wifi Shell Behavior

The WiFi shell prompt is $, while the wired USART1 prompt is shell>. WiFi command lines are parsed separately from the wired shell input buffer, but dispatch uses the same command table through shell_dispatch().

wifi_service() collapses CR/LF pairs, handles backspace/delete, ignores tab/escape, and accepts printable ASCII up to SHELL_MAX_CMD_LEN - 1 characters. If incoming text contains ERROR, firmware treats it as evidence that the module reset back into AT mode and calls wifi_init() again.

The commands idata and who defer the WiFi prompt because their task handlers write directly to WiFi and then print a prompt after the response. Other commands receive a $ prompt immediately after dispatch.

## Realtime ‘idata’ Format

realtime_comm_build() creates a compact CSV payload after profile normalization. It stores the payload in RAM2, computes rt_len, computes a CRC, and marks the payload as unsent. realtime_comm_stream() sends the payload only once; a second idata request before another profile returns “No_Data”.

Current firmware sends the frame preamble as:

@@@CCCCLLLL<payload>

where CCCC is a four-hex-digit CRC and LLLL is a four-hex-digit payload length. The firmware transmit code uses wifi_printf("@@@%04X%04X", rt_crc, rt_len) to transmit the preamble

The CRC covers the four ASCII length bytes plus the payload bytes. It excludes the @@@ and the CRC field. The algorithm is CRC-16/XMODEM-compatible with init 0x0000, polynomial 0x1021, and two final zero-byte augmentations. The check vector 123456789 is 0x31C3.

### ‘idata’ CSV Payload

Payload header: EP,CD,CT,CC,OT,O2,CH,TB,CD

EP = Unix Epoch Time
CD = CTD Depth
CT = CTD Temperature
CC = CTD Conductivity
OT = Optode Temperature
O2 = Oxygen Concentration
CH = Wetlab Chlorophyll
TB = Wetlab Turbidity
CD = Wetlab CDOM

All values are represented as ASCII encoded 16-bit hex values, with the exception of the UNIX Epoch time which is 32-bit. The following measurements are scaled and cast from float to int16:

CD = pressure * 100 (int16)
CT = temperature * 1000 (int16)
CC = conductivity * 10000 (int16)
OT = optode temperature * 1000 (int16)
O2 = O2 concentration * 100 (int16)

# Low-Power State

The low-power manager owns runtime peripheral shutdown, idle timers, STOP2 entry, and wake recovery. Default policy is to keep the system interactive after boot, then enter STOP2 automatically after idle time unless a service is running.

Current low-power timing constants are:

| Constant | Value | Meaning |
| --- | --- | --- |
| LOWPOWER_IDLE_TIMEOUT_SECONDS | 60 | Idle time before entering STOP2 |
| LOWPOWER_STOP2_RETURN_TIMEOUT | 5000 | Wake time (ms) for wifi broadcast |
| LOWPOWER_RTC_WAKE_SECONDS | 20 | RTC wake-up timer |

The system begins its automatic sleep-wake duty cycle when the following are true:

- the idle timer is active

- no sleep request is already pending/prepared

- profile peripherals are down

- g_app.mode is IDLE

- PB8 start is not pending

- the task queue is empty

- no realtime idata payload is waiting to be sent.

If one of those conditions is false, the idle timer start tick is refreshed so the system gets a full idle period after the blocker clears.

## Idle and Profile Peripheral Bring Up/Down

lowpower_idle_peripherals_down() unmounts the filesystem, powers SD/SPI1 down, parks RTC SPI2, powers CTD/Optode/WetLab UART paths down, and marks profile peripherals as down.

lowpower_profile_peripherals_up() performs the profile acquisition recording setup: unmount filesystem, power SD down, stop WiFi, restore RTC SPI2, and power CTD/Optode/WetLab UART paths up.

Profile acquisition and WiFi are mutually. After completion or failure, recorder returns to idle with WiFi on or with a short duty-cycle window depending on the path.

## STOP2 Low-Power Sleep

Before the system enters STOP2 low-power sleep, lowpower_prepare_for_sleep() is called and does the following:

- Checks the current state of the peripherals.

- Sets a 20 second timer on the RTC.

- Refreshes the watchdog timer.

- Unmounts the filesystem if it’s mounted.

- Drives CLKOE pin low.

- Powers down the SPI and UART peripherals as well as the RS232 transceivers.

- Clears any peripheral NVIC bits and checks PB8 in case a profile started.

- Sets PB8 (inductive charger) and PB10 (RTC alarm) as wake signals.

- Enters STOP2 with WFE.

While in STOP2 sleep, the board draws approximately 8uA. While in the IDLE sleep-wake duty cycle, the system spends 20 seconds in STOP2 sleep and 5 seconds awake broadcasting the WIFI AP.

From the shell, the ‘stayawake’ debug command will prevent the system from entering low-power mode and the ‘lowpower-timer’ command will restart the timer.

# Appendix A – Building and Flashing

The firmware is a CMake project generated by STM32CubeMX. Building and flashing requires the STM32 Command Line Tools (CLT) to be installed.

For a debug build, use the following commands:

cmake --preset Debug

cmake --build --preset Debug

Flash the device with the following command:

cmake --build --preset Debug --target flash

# Appendix B – Local CSV and ‘idata’ CSV Formatting

# Appendix C – Client-Side Wifi-Module Setup